# Flooding Triangulated Terrain

Yuanxin Liu 	Jack Snoeyink
Dept. Computer Science, UNC Chapel Hill  
{liuy,snoeyink}@cs.unc.edu

Abstract

We extend pit filling and basin hierarchy computation to TIN terrain models. These operations are relatively easy to implement in drainage computations based on networks (e.g., raster D8 or Voronoi dual) but robustness issues make them difficult to implement in an otherwise appealing model of water flow on a continuous surface such as a TIN. We suggest a consistent solution of the robustness issues, then augment the basin hierarchy graph with different functions for how basins fill and spill to simplify the watershed graph to the essentials.  Our solutions can be tuned by choosing a small number of intuitive parameters to suit applications that require a data-dependent selection of basin hierarchies.

1 Introduction and Previous Work

Without a doubt, the computation of drainage characteristics, including basin boundaries, is one of the successes of GIS analysis. Digital data and GIS are widely employed to partition terrain into hydrological units, such as watersheds and basins. The US proposal for extending the hydrologic unit mapping from a 4-level to a 6-level hierarchy [5] includes discussion of the role of DEMs in what was formerly a manual cartographic process.

A GIS can give preliminary answers to analysis questions such as how much rainfall runoff a downstream point can receive. Detailed hydrologic analysis will apply numerical computation to hillslope patches created to have uniform slope, soil, and/or vegetation characteristics.

The raster DEM is the most common terrain and hydrology model in GIS, because regular grid representations of terrain are most amenable to computation. Many algorithms have been proposed for pit-filling [8,9,10], barrier-breaching [16], flow direction assignment [2,15], basin delineation [12,14,23,24], and other steps of hydrologic modeling [4,13].

Another common terrain model is the TIN (Triangulated Irregular Network), which forms a continuous surface from triangles whose vertices are irregularly-sampled elevation points. A TIN is more complex to store and manipulate because it must explicitly store the irregular topology. The debate between using grid or TIN as the DEM model has been a long-standing one in GIS [11]. Often-mentioned advantages for a TIN include:

- A grid stores the terrain at uniform resolution, while a TIN can potentially store large flat regions as single polygons.

- To construct a grid from irregularly spaced data or multiple source data often requires interpolation and possible loss of information.

- Grid squares do not give a continuous surface model, so geometric operations and measurements on a surface must be approximated on grid cells.

- Water flow on a grid is commonly constrained to eight (or even four) neighbor directions for ease of computation, which can produce visible artifacts.

Our work focuses on basin computations on a TIN, including filling spurious pits and simplifying the basin hierarchy. We provide a framework for basin computation that can support several natural variations efficiently and robustly. We partition the computation into several steps:

- Assign low directions to triangles.  For this we use steepest descent directions on the triangles, but other choices are possible, especially in flat portions of terrain. Triangles can be subdivided or otherwise parameterized to allow more complex specification of flow directions.

- Trace flow paths network. A key contribution of this work is to  keep explicit track of the order that paths cross triangle edges and triangles, and thus avoid computational failures due to degenerate configurations and floating point inaccuracies, which otherwise make this a difficult task.

- Compute basins, spill points, and an initial basin hierarchy.

- Compute basin characteristics such as spill times, by propagation through the basin hierarchy. With consistent flow paths, these become simple problems on graphs. The initial basin hierarchy contains many spurious pits, but its structure can help us decide which are spurious, and which are significant.

- Simplify the basin hierarchy, and, if desired, the terrain to match.  We give examples of hierarchies from basin spill times computed from projected area and volume for basins and the sub-basins that spill into them. Natural variants for computing basin spill times that can be suitable to different type of terrain.

Assignment of flow directions (step 1) and computation of basin spill times (step 4) can be carried out in several ways, a few of which we demonstrate. Our framework calls for simple output of these variants (an assignment of direction vectors to triangles, or event times to basin hierarchy edges) that allows experimentation with algorithm or terrain-dependent policies, while the bulk of the geometric computation remains fixed.

The basin hierarchy has been studied in grid-based terrain by Band et al. [12]. An explicit representation of the basin hierarchy during pit-filling provides structural information that can help us decide which pits are spurious. Moreover, it simplifies the handling of pits—e.g., we need not assign flow directions at spill points, as many pit-filling algorithms on a grid must do, because we just send overflow into the basin we spill into.

Some other researchers have used a TIN as the terrain model for flow computation, but directed flow only along triangle edges, using either the triangles [20,21] or the dual Voronoi cells [18,22] as the hydrological unit. Thus, they still use a discrete network flow model, much like the raster; they simply substitute the regular grid for an irregular network. The flow model that we use has an underlying continuous surface, discretized along flow paths, rather than by a limited set of directions (D8 or edges).

Two sections give definitions and sketches of the algorithms in our work: Sec. 2 reviews previous results [17,25] that we use directly and Sec. 3 describes the basin filling model. Sec. 4 details our implementation, which includes the computational steps above. Sec. 5 describes our experiments, and Sec. 6 discusses possible future work.

2 TIN-based hydrology model

We use a TIN-based hydrology model proposed by Yu et al. [25] that extends the accurate definitions of Frank et al. [6]. We briefly review relevant definitions, properties, and algorithms. A terrain is mathematically modeled as the graph of a real-valued function f over a bounded region of the plane. This definition does not tell us how to store f or, given a set of noisy or irregular data samples, how such a function f might be computed. Still, this general definition of terrain allows us to specify water flow precisely: the trickle path of a point is the steepest descent path that ends either at a local minimum (pit) or the boundary of the terrain (Figure 1c). The watershed of a point p is the set of all points whose trickle paths contain p. The watercourse network is all points whose watersheds have non-zero area. Catchments (strips draining into a portion of the watercourse network) and basins (watersheds of pits) can also be defined.

Since a TIN is a piecewise linear surface, the only segments on a watercourse network are local channels (Figure 1a) and segments of steepest descent paths on triangle faces traced from saddle vertices of the TIN. With this observation, the watercourse network is straightforward to compute: take the set of all segments that belong to the watercourse network and join them at intersection points. It can be easily shown that this watercourse network can be characterized:

- The watercourse network in a TIN is a collection of disjoint (graph-theoretic) trees rooted at pits, whose leaves are local channels.

Figure 1 The thick edges in a) and b) are a local channel and a local ridge. c) shows a trickle path. d) and e) show segments of a watershed graph (dotted) before and after being joined in the neighborhood of a vertex.

McAllister [17] shows how to compute a basin graph: an embedded planar graph for the entire TIN whose faces are polygons that define basins and whose edges separate pairs of adjacent basin faces.  We first compute a watershed graph, which is essentially a basin graph with extra interior segments. We then delete the interior segments to obtain the basin graph. The computation of the watershed graph consists of two steps:

- Collect the local ridges (Figure 1b) of the TIN, and the steepest descent paths traced backward from each saddle point of the TIN.

- The open line segments from step 1 are connected to form an embedded planar graph consistent with the TIN hydrology model.

Step 1 is simple, and we note the similarity between the watershed graph and the watercourse network, whose segments are local channels and steepest descent paths. Intuitively, the segments for the watershed graph form ridges that potentially separate basins.

Step 2 is complicated, and we examine the details here. There are two cases: we can connect the upper end point of a steepest descent path to a local ridge segment, or connect two segments ending at a TIN vertex. The first case is general, and easy to handle. The second is degenerate, and requires care. Figure 1d shows the neighborhood of a vertex, with the open segments shortened by an infinitesimal length. If we simply join these segments by extending them to the vertex, the trickle paths through the vertex will be cut off, dividing a basin face and of the watershed graph. Instead, we must join the segments so they do not “collide” with the trickle paths through the vertex as in Figure 1e. These joining operations in a vertex neighborhood involve only graph operations on data structures.

A key problem with many hydrology models, including this one, is selecting appropriate scale. How can we consistently extract hydrological objects at a desired scale—say, the watershed of a large river—no matter how detailed our terrain model is? With a TIN, two triangles define a segment of the watercourse network regardless of whether they correspond to the banks of a river or sides of a ditch. This problem is exacerbated by data resolutions ranging from 100-meter DEMs to sub-meter LIDAR.

Another key problem of this model is its assumption that water has no volume.  In heavy rainfall, depressions such as storm ponds in urban areas, become inundated, changing the drainage characteristics of the terrain.

The good news is that the work on TIN-based hydrology models allows for an extension that meets the challenge of these key problems.

3 The basin filling model

Suppose that we change the model so that water has volume and can accumulate in basins. Then we must allow a basin to fill until it “spills” into a neighboring basin. Consider the moment the surface of the accumulated water in a basin A reaches the lowest point, a, on its boundary—the spill point. Let basin B be the basin that a drains into. Then the two basins can be merged into a single one: the boundary between A and B are deleted, and the pit of the new basin is the pit of B.

This filling model naturally defines a sequence of merges forming a basin hierarchy tree: leaves are the initial basins, and the root is the merge of all basins. It also defines a sequence of deformation operations on the terrain, each replacing a filled basin by the flat surfaces of its water body. Although trickle paths on flat surfaces are not well defined, since steepest descent is not unique, the trickle path of point p in a filled basin A starts at p and exits at the spill point of A. Therefore, on a “flooded” terrain, a trickle of water can still be directed from its origin to a pit.

To incrementally compute the basin hierarchy tree, we can compute the “spill time” of each basin, then repeatedly take the basin with the earliest spill time, merge it with a neighboring basin and compute the new spill time of the merged basin. This “flooding simulation” algorithm responds to events rather than simulating flow at regular time steps. Each iteration processes one event when a topological change of the hierarchy tree and terrain occurs. The algorithm calculates volumes and surface areas—which can be done accurately up to round-off errors—and updates data structures that store local geometry and connectivity as described in the next section.

The spill times merely define an ordering of the basins; other definitions of spill times can be substituted. We describe experiments in Sec. 5.

4 Data structures and implementation

We have implemented the basin filling computations in C++. Our program takes a set of points in 3D as input and processes them as the following:

- Create a TIN by computing the Delaunay triangulation of the input points (using only the x and y coordinates).

- Compute the watershed graph of the TIN, and delete the internal edges of the graph to create a basin graph.

- Run the flooding computation until one basin is left. At each step, a filled basin is found that spills into a neighbor by merging the corresponding faces in the basin graph. The terrain surface is modified to reflect this by simply deleting the set of triangles that are below the water surface and recording the water surface height with the partially immersed triangles.

- Simplify the basin hierarchy by removing edges, merging pairs of basins, and displaying the corresponding basin graph.

The two most important objects in the program, the TIN and the watershed graph, can each be stored as planar subdivisions using data structures such as Quadedge [7]. However, a number of options should be considered to reduce the space and speed up the computation. These are particularly important due to the size of the data sets we would like to handle.

The watershed graph shares many edge segments with the TIN—in particular, all the local ridges of a TIN are part of the watershed graph. Therefore, instead of duplicating the connectivity information in the TIN, we simply store it in a simple graph data structure with pointers to the segments and vertices of the TIN. An advantage of using a simple graph data structure is that vertex and edge insertion and deletion operations have lower overhead than the same operations on a subdivision. Note, however, that the watershed graph is more than a subset of the TIN. As discussed earlier, the watershed graph in an infinitesimal neighborhood of a saddle point can be very different from the TIN.

If we wish to implement sophisticated subdivision operations such as the merge-face operations for flooding, we want to have the basin graph stored as a subdivision independent of the TIN. Fortunately, the watershed graphs have a large number of internal edges, which implies that the basin graphs are much smaller. We create the basin graph subdivision after we have deleted all the internal edges from the watershed graph. The flooding computation need not know the connectivity between triangles in the TIN. It is enough to keep, for each basin, the set of triangles in it, ordered by the height so that the triangles within a height interval can be quickly located.

Other options that trade between computation time, space, and ease of implementation are yet to be explored. E.g., computing the watershed is currently the memory bottle-neck of our program; pointers between the TIN elements and the simple watershed graph data structure still take too much space. These pointers provide topological information through the TIN that represent the planar embedding of the watershed graph. We can try to reconstruct the basin graph subdivision using only the coordinate information, but numerical errors and degeneracy make this a challenge.

4.1 Robustness issues in implementation

Geometric algorithms that manipulate topological data structures are harder to implement robustly than algorithms that manipulate rasters. The culprits are round-off error and geometric “degeneracies” in a problem. Both have been extensively studied in the field of computational geometry [1,3,19]. Although these results are often quite technical, the most important techniques are not hard to understand. Bit complexity analysis often involves only back-of-the-envelope calculations, and degeneracies can be eliminated by implementing a small set of policies that conceptually perturb the input. We look at these two techniques in more detail to robustly implement the algorithms for this flooding model.

4.1.1 Numerical issues

Geometric algorithms derive spatial relationships from computations on coordinates, but a computer uses only a limited number of bits to represent a number. If algorithm correctness depends on ideal geometry, round-off error results in at best an approximation and, at worst, a crash. Three numerical computations are involved in our algorithm:

- Computing the steepest descent direction for each triangle.

- Testing whether water on some triangle flows into a triangle edge or away from it. This will classify whether an edge is a local channel, a ridge, or neither.

- Tracing the steepest descent path backward from a vertex to a local ridge.

For 1), observe that if we replace “steepest descent” by “unique descent direction,” in the definition of trickle path, the definitions for watersheds are still consistent. So, if we assign each triangle some descent direction that closely approximates the steepest descent, we have a TIN whose drainage characteristics are acceptable as a close approximation of the original. We store the steepest descent vector as single-precision, though the exact vector requires double-precision. We simply round off the directional vector so that the result gives a descent direction.

For 2), testing the flow direction on a triangle edge is an orientation test, which is of algebraic degree two. Since double precision is supported in hardware, an exact implementation of this test is straightforward.

For 3), we have the most problematic numeric computation involved in the algorithm. Each time we cross a triangle as we back-trace a steepest descent path, we must compute the intersection of the steepest ascent path with an edge of the triangle, which quadruples the precision for exact computation. If k intersections are computed, we go from b bits to a worst case of  (3k+1)b  bits to represent the last point of the path exactly. We use single precision floating point instead, with the unfortunate consequence that not only does the path's position in space become inexact, we can no longer guarantee watershed graph properties such as that each face contains exactly one pit. This approximation can be acceptable as long as small errors do not cause catastrophic changes. The main issue is that two inexact paths might cross each other, contradicting the assumption that the steepest descent path is unique. We can try two ways to handle this:

- We can compute the watershed graph without regard to whether the steepest ascent paths cross each other. Then, the nodes of the computed watershed graph that are supposed to be on the steepest ascent paths cannot be embedded right away. We must first repair steepest ascent paths so they no longer intersect.

- We can incrementally maintain the invariant that no steepest ascent paths cross by asserting that the order of the entry points of the steepest ascent paths into a polygon must be the same as the order of the exit points. In the data structure, we maintain a list of all steepest ascent flows across each triangle edge.

- We have chosen 2) in our implementation. When we back-trace a steepest descent path through a triangle, we first compute the position of the exit point by numeric computation. If this position fails to satisfy the invariant, we assign a position in the valid range that the path is allowed to exit. In our experiments, we have found that the number of faces that do not have exactly one pit is small and the deviation is never more than one.

4.1.2 Resolving degeneracies

Geometric algorithms often assume that the inputs are in general position. For example, no three points lie on the same line. Subsets of the input that violate the assumption are called degeneracies [1] because an infinitesimal random perturbation of the input will eliminate them. To actually handle the degeneracies in an implementation, one can either directly handle all the “exceptions” introduced, or create policies that treat the degeneracies consistent with some infinitesimal perturbation. In McAllister [17], the first option is taken, while we have done the latter. We list below the general position assumptions made in our algorithm and how the corresponding degeneracies are handled with perturbations.

- No two points have the same height.  In the degenerate case, when two z-values are equal, compare their x-values; and if their x-values are equal, compare their y-values. This policy is consistent with infinitesimally rotating the xy-plane.

- The steepest descent direction in a triangle is not in the same direction as any of the edge of the triangle. We need this to test whether water on a triangle flows into a triangle edge or away. In the degenerate case, we choose “away.” This is consistent with rotating the steepest descent direction infinitesimally.

- The steepest ascent path does not go through a triangle vertex.  In the degenerate case, we choose one of the two edges adjacent to the vertex as the next edge. This is consistent with perturbing the x y coordinates of the vertex infinitesimally. Note that this perturbation precludes the possibility that an area can drain into another area through a single trickle path.

When implementing a perturbation policy, we must reason backwards about what the policy implies to how the input is perturbed and convince ourselves that perturbations in different operations are not contradictory. If this is not done, the behavior of the program can be unpredictable.

5 Experiments

We have tested our program against digital elevation data from various sources. The graphical output of the program is shown in Figure 2. Basin boundaries are drawn as outlines over the hill-shaded terrains. Black regions are flooded regions of the terrain.

We have used several different basin filling strategies. Each strategy effectively defines a different basin spill time. We have experimented with these four strategies:

- Uniform precipitation model: compute a basin's filling time by dividing the volume of a basin by (projected) area. Once a basin spills into another, the areas and volumes add. This is consistent with the simplistic, but physical, assumption  that all rainfall turns into surface flow.

- The basins are filled in order of increasing volume.

- The basins are filled in order of increasing area.

- For each basin, we compute two numbers: the number of basins that have spilled into it and its filling time under the uniform  precipitation model. Two basins are compared by lexicographic ordering of their number pairs.

These filling strategies—some corresponding to precipitation models—demonstrate the flexibility of the flooding model and the program.  The program stores enough topological and geometrical information so that the numbers used for the ordering can be easily computed.

Figure 2 shows the output using the strategies 1 and 2 (please refer to our website, http://www.cs.unc.edu/~liuy/flooding/, for more results); two terrain models were used as the input. The terrain model for the first column of pictures was produced from 1 degree quad of USGS DEM data from Grand Junction, Colorado. The second terrain model was produced from unfiltered LIDAR data from Baisman Run, Maryland.

These particular filling strategies were motivated by observations about our initial results from a uniform precipitation model. For example, in the second row from Figure 2, we see that large rivers merge before small basins are filled. This is unfortunate, though unsurprising with this model, since a large basin with a low spill point holds a relatively small amount of water compared to the amount of water it receives through precipitation over a large area. Our other filling strategies somewhat alleviate this problem. The improvement is especially obvious with LIDAR data that contains elevation points from vegetation that often produce “wells” on the terrain that are hard to fill with a uniform precipitation model.

6 Conclusions and Future Work

We have shown how to produce basin hierarchies on a TIN with the basin filling model. We have implemented our algorithms robustly and given sample outputs. Currently, the largest TINs we can handle are about one million data points, so we would like to use more out-of-core algorithms and data structures to handle tens of millions of points. We would like to compare our results against basin hierarchies produced with grid-based algorithms, and to include more physically-based parameters into our implementation, such as absorption rate determined by land cover type.

We would also like to incorporate the basin graph into TIN simplification so that the basin graph of the simplified terrain is close to a basin graph at some appropriate level of the basin hierarchy of the original terrain, where closeness is defined both topologically and geometrically.

References

[1] P. Alliez, O. Devillers, and J. Snoeyink. Removing degeneracies by perturbing the problem or perturbing the world. Reliable Computing, 6:61-79, 2000.

[2] T-Y Chou, W-T Lin, C-Y Lin, W-C Chou, and P-H Huang. Application of the PROMETHEE technique to determine depression outlet location and  flow direction in DEM. J Hydrol, 287(1-4):49-61, 2004.

[3] H. Edelsbrunner and E.P. Mücke. Simulation of simplicity: A technique to cope with degenerate cases in geometric algorithms. ACM TOG., 9(1):66–104, 1990.

[4] J. Fairfield and P. Leymarie. Drainage networks from grid digital elevation models. Water Resour. Res, 27:709–717, 1991.

[5] Federal standards for delineation of hydrologic unit boundaries. version 1.0. http://www.ftw.nrcs.usda.gov/HUC/HU_standards_v1_030102.doc, mar 2001.

[6] A.U. Frank, B. Palmer, and V.B. Robinson. Formal methods for the accurate definition of some fundamental terms in physical geography. In Proc. 2nd Intl. SDH, pages 585–599, 1986.

[7] Leonidas J. Guibas and J. Stolfi. Primitives for the manipulation of general subdivisions and the computation of Voronoi diagrams. ACM TOG, 4(2):74-123, 1985.

[8] M.F. Hutchinson. Calculation of hydrologically sound digital elevation models. In Proc. 3rd Int. SDH, pages 117–133, 1988.

[9] S.K. Jenson and J.O. Domingue. Extracting topographic structure from digital elevation data for geographic information system analysis. Photogrammetric Engineering and Remote Sensing, 54(11):1593–1600, Nov. 1988.

[10] S.K. Jenson and C.M. Trautwein. Methods and applications in surface depression analysis. In Proc. AUTO-CARTO 8, pages 137–144, 1987.

[11] M.P. Kumler. An intensive comparison of triangulated irregular networks (TINs) and digital elevation models (DEMs). Cartog., 31(2), 1994. mon 45.

[12] D.S. Mackay and L.E. Band. Extraction and representation of nested catchment areas from digital elevation models in lake-dominated topography. Water Resources Research, 34(4):897–902, 1998.

[13] David R. Maidment. GIS and hydrologic modeling – an assessment of progress. In Proc. Third Conf. GIS and Environmental Modeling, Santa Fe, NM, 1996. NCGIA http://www.sbg.ac.at/geo/idrisi/gis_environmental_modeling/ sf_papers/maidment_david/maidment.html.

[14] D. Marks, J. Dozier, and J. Frew. Automated basin delineation from digital elevation data. Geo-Processing, 2:299–311, 1984.

[15] L.W. Martz and J. Garbrecht. The treatment of flat areas and closed depressions in automated drainage analysis of raster digital elevation models. Hydrological Processes, 12:843–855, 1998.

[16] L.W. Martz and J. Garbrecht. An outlet breaching algorithm for the treatment of closed depressions in a raster DEM. Computers and Geosciences, 25, 1999.

[17] M. McAllister. The Computational Geometry of Hydrology Data in Geographic Information System. Ph.D. thesis, UBC CS, Vancouver, 1999.

[18] O.L. Palacios-Velez and B. Cuevas-Renaud. Automated river-course, ridge and basin delineation from digital elevation data. J Hydrol, 86:299–314, 1986.

[19] J. Shewchuk. Adaptive precision floating point arithmetic and fast robust geometric predicates. Discrete & Comp. Geom. 18:305-363, 1997.

[20] A.T. Silfer, G.J. Kinn, and J.M. Hassett. A geographic information system utilizing the triangulated irregular network as a basis for hydrologic modeling. In Proc. Auto-Carto 8, pages 129–136, 1987.

[21] D.M. Theobald and M.F. Goodchild. Artifacts of TIN-based surface flow modeling. In Proc. GIS/LIS’90, pages 955–964, 1990.

[22] G.E. Tucker, S.T. Lancaster, N.M. Gasparini, R.L. Bras, S.M. Rybarczyk. An object-oriented framework for distributed hydrologic and geomorphic model-ing using triangulated irregular networks. Comp Geosc, 27(8):959–973, 2001.

[23] K. Verdin and S. Jenson. Development of continental scale DEMs and extraction of hydrographic features. In Proc. 3rd Conf. GIS and Env. Model., Santa Fe, 1996. http://edcdaac.usgs.gov/gtopo30/papers/santafe3.html.

[24] J.V. Vogt, R. Colombo, and F. Bertolo. Deriving drainage networks and catchment boundaries: a new methodology combining digital elevation data and environmental characteristics. Geomorph., 53(3-4):281–298, July 2003.

[25] S. Yu, M. van Kreveld, and J. Snoeyink. Drainage queries in TINs: from local to global and back again. In Proc. 7th SDH, pages 13A.1–13A.14, 1996.

Figure 2 Basin graphs from two terrain models using two filling stratgies.  Each basin is delineated. The number of basins is also shown.

Research partially supported by NSF grant 9988742.